# Introduction

The idea of this BASIC simulation program is to give the complete beginner an idea of how a microprocessor works in terms of data processing and movement. In particular it demonstrates that any given byte of data in the processor's memory could be

a) a coded instruction,

b) part of the address of some other part of memory, or

c) some actual data which is used in the course of the processing.

The simulated processor has 50 'bytes’ of memory, each of which can contain a single decimal digit. The contents of memory are constantly visible, as are the accumulator and carry, and input and output ports.

The set of instructions is limited to ten, most of which are related to 6502 instructions, but if necessary the ten that are used to run any given program may be changed. The debugging of programs is greatly simplified by the ability to execute programs a step at a time, with the cursor program counter) indicating the next instruction to be interpreted.

input port	output port

Display

Ready error		READY 	INPUT 	PORT	OUTPUT 	PORT

### Program mode		PROC	     98         0	 	         99           0

### Run mode		RUN

### LDA 23		CARRY     0 	ZERO          0 	ACC     7

Instruction		0	 1  	3  	2  	1   	1  	5  	5 	0  	9	3

mnemonic

1	5	6	6	2	0	6	0	9	8	5

Cursor shows

current	2	7	4	1	8	9	6	0	2	9	2

location

3	6	1	0	1	2	3	1	2	7	1

High - order

address	4	1	1	2	5	3	4	8	6	9	4

digit

0	1	2	3	4	5	6	7	8	9

low – order address digit

# Once the PEEKO - Computer and all its features have been mastered it is quite an easy step to write machine code programs in 6502 code, opening up a whole new range of fast - action applications for the ATOM.

# PEEKO - Computer Monitor

When PEEKO is RUN the user enters the ‘monitor’ of the PEEKO - computer. The monitor commands are single keystrokes:

0 - 9	Pressing any digit key will modify the contents of the memory location indicated by the cursor; the cursor then moves on to the next location. The monitor replies with a bleep to show that it has accepted the data, thus making it easy to enter programs without looking at the display. The mnemonic of each instruction being entered is shown as it is built up, with the format of any arguments expected.

G (GO)	Causes the PEEKO - computer to execute the program in its memory, starting at location zero. The program will continue to run until it meets a BRK instruction, or the user presses E (escape).

F (FAST)	Similar to C, but the program runs faster.

E (ESCAPE)	In RUN mode S stops the execution of the program and returns to READY mode.

SPACE	Allows the program to be executed one step at a time. By repeatedly pressing space successive instructions are executed, and the mnemonic for each instruction is displayed.

The ATOM’s usual cursor controls can be used to move the cursor around.

O (ORIGIN) 	Moves the cursor back to memory location zero.

S (SAVE)	Allows programs to be saved in the ATOM’s graphics memory. The S is followed by a single digit, 0 - 9, which identifies the program. If only 3K of graphics memory is present only

0  to 3 should be used.

L (LOAD)	Followed by a single digit, it allows the corresponding program to be loaded from the ATOM’s memory into the PEEKO - computer.

I	Allows the instruction set of the PEEKO - computer to be changed. In addition to the 10 standard instructions there is a second set of 10 instructions, any of which can be substituted for any of the standard ones. In this mode entering S will return the PEEKO - computer to its standard instruction set.

P (PRINT)	Dumps the program in the PEEKO - computer onto a printer. The dump will be printed in double - width characters on an Acorn 02 - 80; for other printers modifications to lines 9400 to 9410 may be necessary.

N (NEXT)	Steps through the program, an instruction at a time, displaying each mnemonic -

>	Saves the program, and the current instruction set, onto cassette. It will first prompt for a filename.

< 	This will load a program and instruction set from cassette having first prompted for the filename.

## An Introduction to Microprocessors

What do microprocessors do?

1.	A microprocessor uses a number of memory ‘locations’, each of which contains a ‘byte’. In the PEEKO - computer each byte is a single decimal digit.

2.	Each memory location has a unique ‘address On the PEEKO - computer the address consists of two decimal digits; reading left to right, row by row, the addresses are 00, 01, 02          ……… 48, 49.

3.	The byte at each location can represent either:

(a)	an ‘instruction’, On the PEEKO - conputer there are ten different instructions, 0 to 9;

(b)	part of an address  -  two bytes are required to specify an address;

(c)	some ‘data’ which the processor can use and/or modify during the course of its program.

4.	The processor itself contains ‘registers’ which it may use during the course of a progran. The PEEKO - computer has only one  -  the ‘accumulator’

5.	Some special addresses can be used to pass information to and from the outside world, using the ‘output port’ and ‘input port’. On the PEEKO - computer the input port has address 98 and the output port address 99.

Instructions

Each instruction has a name of three or four letters called the ‘mnemonic’ which describes what the instruction does.

Different instructions require different information following them:

(a) 	Some require two bytes to specify the address of the memory location to which the instruction refers. For example, the ‘store accumulator’ instruction, STA, is followed by the address where the byte in the accumulator is to ho

stored:

2   |  0  |   4

|	|

INSTRUCTION	ADDRESS

STA	40

It is important to note that in the PEEKO - computer the low byte of the address MUST be specified FIRST, although the instruction mnemonic shows the bytes the usual way round.

- Some instructions are ‘immediately’ followed by a single byte which is the data to be used by the instruction. For example, the ‘load accumulator immediate’ instruction, LDA @ is followed by the number to be loaded:

7  |  5

|	|

INSTRUCTION	DATA

LDA@	5

(c) 	Some instructions do not require any data. For example, the ‘clear carry’ instruction, CLC, simply clears the carry flag to zero:

| 3  |

|

INSTRUCTION

CLC

Instruction set

The standard PEEKO - computor instruction set is:

Mnemonic: 				Description:

0	BRK		Break from program to monitor

1	LDA	xx	Load accumulator from location xx

2	STA	xx	Store accumulator to location xx

3	CLC		Clear carry to zero

4	ADC	xx	Add the contents of location xx to the accumulator with carry

5	DEC	xx	Decrement the contents of location xx

6	INC	xx	Increment the contents of location xx

7	LDA@ 	x	Load accumulator immediately with x

8	JMP	xx	Jump to xx

9	JNE	xx	Jump to xx if previmus result not equal to zero; zero flag clear

Programs

A ‘program’ consists of a sequence of instructions which the processor executes, instruction by instruction, starting from the lowest address until it reaches a BRK instruction. The ‘jump’ instructions are designed to make the processor leave its normal sequence of execution, and fetch its next instruction from some other specified address.

## Programming the PEEKO - Computer

The following section illustrates the use of the PF.EKO computer’s instructions. Most of the instructions will be introduced by means ef a demonstration program. Each program will be shown as a series of mnemonics and their assembled code, and in most cases a printout of the screen.

Example 1  -  Adding two numbers

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

LDA 40		CARRY 0 	ZERO 0 	ACC 7

0		1   	0  	4  	3   	4  	1  	4  	2  	2	4

1	0	0	0	0	0	0	0	0	0	0

2	0	0	0	0	0	0	0	0	0	0

3	0	0	0	0	0	0	0	0	0	0

4	3	4	7	0	0	0	0	0	0	0

0	1	2	3	4	5	6	7	8	9

Loc.	Mnemonic		Code
	00	LDA	40		1 0 4
	03	CLC			3
	04	ADC	41		4 1 4
	07	STA	42		2 2 4
	10	BRK			0

First type in the code starting at location 00. Notice how each instruction is ‘built - up’ on the line below ‘RUN’ on the screen. Next enter the two digits to be added at locations 40 and 41  -  use the cursor controls to get the cursor there.

To run the program type space to enter single step mode. Pressing space again executes the first instruction, LDA 40; this loads the contents of location 40 into the accumulator. The next instruction, CLC clears the carry flag to zero. The ADC adds the contents of 41 to the accumulator with the zero carry and puts the result in the accumulator. Next the STA instruction will store the result in location 42. Finally BRK causes the program to stop.

If the numbers chosen added up to 10 or more it will be noticed that 42 only contains the last digit and the carry is set to 1. The next example uses the carry flag to add two two - digit numbers.

By typing C or F from the monitor the program can be run without single stepping.

Example 2 - Add 2 two - digit numbers

This adds the numbers in 40, 41 and 42, 43 and puts the result in 44, 45, 46.

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

CLC		CARRY 0 	ZERO 0 	ACC 1

0		3	1	1	4	4	3	4	2	6	4

1	1	0	4	4	2	4	2	5	4	7

2	0	4	0	2	2	4	4	0	0	0

3	0	0	0	0	0	0	0	0	0	0

4	7	6	6	1	1	3	7	0	0	0

0	1	2	3	4	5	6	7	8	9

Loc. Mnemonic 	Code

00	CLC		3
01	LDA 	41	1 1	4
04	ADC 	43	4 3	4
07	STA 	46	2 6	4
10	LDA 	40	1 0	4
13	ADC 	42	4 2	4
16	STA 	45	2 5	4
19	LDA @ 0	7 0
21	ADC 	20	4 0	2
24	STA 	44	2 4	4
27	BRK	0

This program contains one new instruction this is LDA @ which loads the accumulator immediately with the next byte in the program  -  in this case 0.

Problem 1

Using LDA. STA, ADC, CLC and LDA@ write a program that will add the numbers in locations 40, 41 and 42 storing the two digit result in 43 and 44.

Example 3  -  Countdown program

The following program will subtract one from location 40 each time it goes round a continuous loop.

(N.b. 0 - 1 = 9)

Loc 	Mnemonic 	Code

00	DEC	40	5 0 4
03	JMP	00	8 0 0

This program introduces two new instructions. DEC subtracts one from the specified memory location

-  in this case 40  -  and JMP causes the program to goto the location specified by the next two bytes

-  like GOTO in BASIC. As this program is a continuous loop E will have to be typed to get out of RUN mode. By typing F instead of space to run the program PEEKO will execute the program without stopping after each instruction.

Example 4  -  Count down from 99

This will count down the two digits in 40 and 41 to zero and then stop.

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

LDA 41	CARRY 0 	ZERO 0 	ACC 1

0		5	1	4	9	0	0	5	0	4	9

1	0	0	0	0	0	0	0	0	0	0

2	0	0	0	0	0	0	0	0	0	0

3	0	0	0	0	0	0	0	0	0	0

4 	9	9	0	0 	0	0	0	0	0	0

0	1	2	3	4	5	6	7	8	9

Loc. Mnemonic	Code

00	DEC	41	5	1	4
03	JNE	00	9	0	0
06	DEC	40	5	0	4
09	JNE	00	9	0	0
12	BRK		0

40			9
41			9

By single stepping through this it will be noticed that when location 41 gets to 0 the zero flag changes from 0 to 1. The zero flag is set whenever an arithmetic instruction gives a zero result, and to 0 otherwise.

The JNE instruction is similar to JMP except that the jump only occurs if the previous result was not equal to zero. This causes the program to count down to zero.

The INC instruction is similar to DEC except that it adds one to the following address. It also will set the zero flag if the result is zero, my replacing DEC (code 5) with INC code 6) in the previous program and setting 40 and 41 to 0 the program can be made to count up to (1)00.

Problem 2

Write a program that will count from 99 to 0 and back up to (1)00 in a continuous loop.

Alternative instructions

The instructions discussed so far form the standard instruction set. In addition to these there are 10 alternative instructions which can replace any of the standard instructions. The (i) command in the PEEKO Computer will ask for the number of the instruction to be replaced and the number of the instruction that will take its place.

This ability to change the instruction set makes the PEEKO - computer very adaptable and able to be programmed for many tasks despite its limited program size. Most of this secondary set can be explained very easily as they are similar to those in the standard set

Alternative instruction set

SEC		Set carry to I
SBC	xx	Subtract contents of xx from the
		accumulator with carry
JCC	xx	Jump to xx if carry is clear
JCS	xx	Jump to xx if carry is set
JEQ	xx	Jump to xx if zero flay is set
DECA		Subtract one from the accumulator
INCA		Add one to the accumulator
CMP	@x	Compare the accumulator immediately with x.

If they are equal set zero flag; otherwise clear zero flag.

LDA 	(xx)	Loads the accumulator with the contents of the address pointed to by locations xx

and xx+1.

SrA	(xx)	works in a similar manner to LDA (xx) storing the result at the location
			pointed to by xx.

The last two extra instructions are an indirect load and store. These are more complex than the others so will be dealt with in greater depth in example 6,

The next 5 programs use some of these features. Before they are typed in the instruction set should be changed to the required set as shown in each example.

Example 5 — subtracting 2 two - digit numbers Instruction set changes:

3 SEC

4 SBC

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

LDA 41	CARRY 1	ZERO 0 	ACC 8

0		1	1	4 	3	4	3	4 	2	5 	4

4	1	0	4 	4	2	4	2 	4	4 	0

2	0	0	0	0	0	0	0	0	0	0

3	0	0	0	0	0	0	0	0	0	0

4	6	7	1	1	5	6	0	0	0	0

^	    -               =         ^

0	1	2	3	4	5	6	7	8	9

Loc. 	Mnemonic 	Code

00	LDA	41	1 1 4
03	SEC		3
04	SBC	43	4 3 4
07	STA	45	2 5 4
10	LDA	40	1 0 4
13	SBC	42	4 2 4
16	STA	44	2 4 4
19	BRK		0

Calculates 40, 41 - 42, 43 with the result in 44, 45. A carry of 1 indicates no overflow. If the carry is 0 then 42, 43 was greater than 40, 41 and hence there is a negative result.

Problem 3

Write a program to multiply two single - digit numbers by successive addition.

Example 6  -  Indirect addressing

Instruction set changes:

1 LDA (

2 STA (

This program uses indirect addressing. If locations 40 and 41 were:      0    2

and location 20 was:     8

then LDA ( 40 ) would load the accumulator from location 20 (as 40, 41 contains 20) and would therefore be loaded with 8.

Loc. Mnemonic 	Code

00	LDA (40)	1 0 4
03	STA (42)	2 2 4
06	BRK

40			0
41			3
42			0
43			2

By running this short program you will be able to see how indirect addressing can be used to move the contents of one address to another address, in this case from 30 to 20.

Example 7 — Copying a 10—digit table

The program for this example is supplied on cassette, as the file COPY, and can be loaded into the PEEKO - computer using the < command.

Instruction set changes:

- LDA  (

- STA  (

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

LDA (40)	CARRY 0	ZERO 0 	ACC 8

0		1	0	4	2	2	4	6	0	4	6

1	2	4	9	0	0	0	0	0	0	0

2	1	2	3	4	5	6	7	8	9	0

^ Copy from here

3	0	0	0	0	0	0	0	0	0	0

^	to here

4	0	2	0	3	0	0	0	0	0	0

0	1	2	3	4	5	6	7	8	9

Loc. Mnemonic 	Code

00	LDA	(40) 	1 0 4
03	STA	(42)	2 2 4
06	INC	40	6 0 4
09	INC	42	6 2 4
12	INS	00	9 0 0
15	BRK		0

40			0
41			2
42			0

43			3

This example shows how indirect addressing can easily be used to access a table of data. The program will copy a lime of data, starting at the location pointed to by 40, 41, to the location pointed to by 42, 43. Locations 20 to 29 should contain the data to be copied to 30 to 39.

Example 8  -  Add two lines of data

This program is supplied on cassette as the file ADD.

Instruction set changes:

1 LDA  (

2 STA  (

7 STA

The program will add two lines of data in the memory together, and store the result. As space is limited in the PEEKO computer the program actually adds up two lines within the program itself.

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

CLC	CARRY 0	ZERO 0 	ACC 5

0		3	1	3	4	7	3	4	1	6	4

+	+	+	+	+	+	+	+	+

1 	4 	3 	4 	2	4 	4 	5 	8 	4 	5

2	6	4	5	4	4	9	1	0	0	0

=	=	=	=	=	=	=	=	=

3	0	5	2	7	1	8	0	0	0	9

4	0	0	0	1	0 	3	0	1	0	0

0	1	2	3	4	5	6	7	8	9

Loc.	Mnemonic  	Code
00	CLC		3
01	LDA	( 48 ) 	1 8 4
04	STA	43 	7 3 4
07	LDA	( 46 ) 	1 6 4
10	ADC	43 	4 3 4
13	SEA	( 44 ) 	2 4 4
16	DEC	48 	5 8 4
19	DEC	46 	5 6 4
22	DEC	44 	5 4 4
25	JNE	01 	9 1 0

28	BRK		0

44			9
45			3
46			9
47			1
48			9
49			0

This program again uses indirect addressing extensively.

Example 9  -  Finding a factor of a number

This program is supplied on cassette as the file FACTOR.

Instruction set changes:

- SEC

- SBC

8  JCC

The program finds the highest factor less than 10 of the 2 digit number in 48, 49.

READY  	INPUT 	PORT	OUTPUT 	PORT

PROC	     98    0		       99    0

RUN

CLC	CARRY 0	ZERO 0 	ACC 5

0		1	8	4	2	8	2	1	9 	4	2

1		0	2	5	7	4	3	8	0	0	7

2	0	4	7	4	2	0	2	7	0	4

3 	6 	4 	2 	8 	2 	9 	6 	1 	1 	0

4	2	9	6	1	0	0	0	8	9 	6

factor ^ number

0	1	2	3	4	5	6	7	8	9

Loc. Mnemonic	Code

00	LDA	48	1 8	4
03	STA	28	2 8	2
06	LDA	49	1 9
09	STA	20	2 0	2
12	DEC	47	5 7	4
15	SEC		3
16	JCC	00	8 0	0
19	LDA	@ 0	7 0
21	SBC	47	4 7	4
24	STA	20	2 0	2
27	LDA	@ 0	7 0
29	SBC	46	4 6	4
Loc. Mnemonic	Code

32	STA	28	2 8	2

35	JNE	16	9 6	1
38	LDA	20	1 0	2
41	JNE	16	9 6	1
44	BRK		0

46			0

47			0

Put the number to be factorised in 48, 49.

Problem 4

Program the PEEKO - computer to divide a 2 digit number by a single digit. This will certainly need SBC and SEC and could also need some other instructions. Take the factor program as an example.

Input and Output Ports

These two ports, at locations 98 and 99 in the PEEKO computer, correspond to the A and B ports of the 6522 VIA chip which can be fitted inside the ATOM. The input port reads in the bottom 4 hits of port A, putting the data in PEEKO location 98. If the Input is larger than 10 it is cut to one digit and the carry flag is set to 1. The output port outputs the data to port B.

Since the printer also uses the 6522 VIA chip for its output, it will be necessary to unplug any I/O devices and plug in the printer if a hard copy of a program using these ports is needed.

Ansvers to Problems

Problem 1

Loc. Mnemonic	Code

00	CLC		3
01	LDA	44	1 0	4
04	ADC	41	4 1	4
07	STA	44	2 4	4
10	LDA	@0	7 0
12	ADC	11	4 1	1
15	STA	43	2 3	4
18	LDA	44	1 4	4
21	ADC	42	4 2	4
24	STA	44	2 4	4
27	LDA	43	1 3	4
30	ADC	11	4 1	1
33	STA	43	2 3	4
36	BRK		0

Problem 2

Loc. Mnemonic	Code

00	DEC	41	5 1	4
03	JNE	00	9 0	0
06	DEC	40	5 0	4
09	JNE	00	9 0	0
12	INC	41	6 1	4
15	JNE	12	9 2	1
18	INC	40	6 0	4
21	JNE	12	9 2	1
24	JMP	00	8 0	0

This proqram will count up and down locations 40 and 41. You will probably notice that it does not

stop at 00 when counting down, but when it decrements from ten (i.e. 12 11 10 00 01 02 .

You should be able to correct this when you have found out about the carry flag and its related

conditional jumps later.

Problem		3
Loc.	Mnemonics	Code
00	LDA	41	1 4 1
03	STA	44	2 4 4
06	CLC		3
07	LDA	40	1 0 4

10 	ADC 	43	4 3 4

13 	STA 	43	2 3 4

16 	LDA 	@ 0	7 0

18 	ADC 	42	2 2 4

21 	STA 	42	2 2 4

24 	DEC 	44	5 4 4

27 	JNE 	06	9 0 6

30 	BRK		0

This multiplies the contents of 40 and 41 puttinq the result in 42 and 43.

Problem 4

instruction set changes:

3 SEC

4 SBC

Loc. Mnemonics 	Code

00	LDA	44	1 4	4
03	SPA	48	2 8	4
06	LDA	45	1 5	4
09	SPA	49	2 9	4
12	INC	47	6 7	4
15	LDA	49	1 9	4
18	SBC		3
19	SBC		4 6	4
22	STA	49	2 9	4
25	LDA	48	1 8	4
28	SBC	40	4 0	4
31	STA	48	2 8	4
34	JNE	12	9 2	1
37	DEC	47	5 7	4
40	BRK		0

This divides the contents of 44 and 45 by the contents of 46, leaving the result in 47, 48 and 49 are used in the calculation.

# PEEKO  Monitor commands

0 – 9	Enter byte into memory

Space	Single step through program

E	Escape from program run

F	Fast execution of program

G	Go – execute program

I	Instruction set

L	Load program from memory

N	Next instruction

O	Origin – cursor to 00

P	Print screen

S	Save program to memory

<	Load program from cassette

>	Save program to cassette

# Instruction set

Mnemonic: 				Description:

00	BRK		Break from program to monitor

01	LDA	xx	Load accumulator from location xx

02	STA	xx	Store accumulator to location xx

03	CLC		Clear carry to zero

04	ADC	xx	Add the contents of location xx to the accumulator with carry

05	DEC	xx	Decrement the contents of location xx

06	INC	xx	Increment the contents of location xx

07	LDA   @ 	x	Load accumulator immediately with x

08	JMP	xx	Jump to xx

09	JNE	xx	Jump to xx if previmus result not equal to zero; zero flag clear

10	SEC		Set carry

11	SBC	xx	Subtract with carry

12	JCC	xx	Jump if carry clear

13	JCS	xx	Jump if carry set

14	JEQ	xx	Jump if equal

15	DECA		Decrement accumulator

16	INCA		Increment	 accumulator

17	CMP  @ x	Compare immediate

18	LDA 	(xx)	Load accumulator indirect

19	STA	(xx)	Store accumulator indirect